## 1. Project Overview

This project implements a smart fire detection system using a Raspberry Pi combined with multiple sensors, AI-based risk prediction, real-time IoT monitoring (Blynk), and SMS alerts (SIM800L GSM).

The system continuously monitors:

- 🔥 Flame presence

- 🌡 Temperature

- 💨 Smoke/Gas concentration (MQ-2)

When a fire risk is detected (by sensors or AI), the system:

- Activates a buzzer alarm

- Sends an SMS alert

- Updates live data on Blynk mobile app

## 2. System Architecture

### Hardware Components

- Raspberry Pi

- MQ-2 Smoke/Gas Sensor (via MCP3008 ADC)

- DS18B20 Temperature Sensor

- Flame Sensor

- SIM800L GSM Module

- Buzzer

- MCP3008 ADC

- External Power Supply

### Software Components

- Python 3

- RPi.GPIO

- Adafruit MCP3008 Library

- NumPy

- Joblib (AI model)

- Blynk IoT Platform

- Multithreading

## 3. Features

✔ Multi-sensor fire detection
✔ AI-based fire risk prediction
✔ Real-time IoT dashboard (Blynk)
✔ SMS alerts with cooldown protection
✔ Buzzer alarm
✔ Threaded architecture (non-blocking)
✔ Automatic MQ-2 calibration

## 4. Blynk Configuration

BLYNK_AUTH = "PUT_YOUR_TOKEN_HERE"

Virtual Pins Used:

| Virtual Pin | Data |
| --- | --- |
| V0 | Temperature (°C) |
| V1 | MQ-2 Smoke Value |
| V2 | Flame Status |
| V3 | AI Risk Probability |
| V4 | Alert Status |

## 5. Configuration Parameters

FLAME_PIN = 17

BUZZER_PIN = 27

SERIAL_PORT = "/dev/serial0"

BAUD_RATE = 9600

PHONE_NUMBER = "01025681976"

TEMP_THRESHOLD = 50.0

SMOKE_FACTOR = 1.5

AI_THRESHOLD = 0.70

| Parameter | Description |
| --- | --- |
| TEMP_THRESHOLD | Max safe temperature |
| SMOKE_FACTOR | MQ-2 smoke multiplier |
| AI_THRESHOLD | Fire risk probability limit |

## 6. Sensor Modules

### 🔥 Flame Sensor

- Connected via GPIO

- Detects direct flame presence

- Digital input (HIGH = fire)

### 🌡 DS18B20 Temperature Sensor

def read_temp():

...

- Reads temperature from Linux 1-Wire interface

- Returns temperature in Celsius

### 💨 MQ-2 Smoke Sensor (MCP3008 ADC)

mq2 = AnalogIn(mcp, MCP.P0)

- Uses SPI communication

- Returns analog smoke concentration

- Values are smoothed using rolling average

## 7. MQ-2 Calibration

def calibrate_mq2():

Process:

- Warm-up for 15 seconds

- Read 50 samples

- Compute baseline

- Define smoke threshold

Threshold = Baseline × SMOKE_FACTOR

## 8. AI Fire Risk Prediction

### Model

- Logistic Regression (joblib file)

- Features used:

| Feature | Description |
| --- | --- |
| Temperature | Normalized |
| MQ-2 Value | Normalized |
| Flame | Binary |
| Smoke Ratio | Current / Baseline |
| ΔTemperature | Change rate |
| ΔSmoke | Change rate |

sensor_data["risk"] = lr_model.predict_proba(X)[0][1]

## 9. GSM SMS Alert System

### SIM800L Configuration

SERIAL_PORT = "/dev/serial0"

BAUD_RATE = 9600

### SMS Protection

- Cooldown: 30 seconds

- Prevents message spamming

SMS_COOLDOWN = 30

## 10. Alert Logic

An alert is triggered if any condition is true:

flame detected

OR smoke > threshold

OR temperature > threshold

OR AI risk ≥ AI_THRESHOLD

Actions:

- 🔊 Buzzer ON

- 📩 SMS sent

- 📲 Blynk alert updated

## 11. Multithreading Design

Each function runs in its own thread:

| Thread | Function |
| --- | --- |
| Flame Thread | Reads flame sensor |
| MQ-2 Thread | Reads smoke values |
| Temp Thread | Reads temperature |
| AI Thread | Predicts fire risk |
| Blynk Thread | Updates IoT + alerts |

Threads are started as daemon threads to ensure clean shutdown.

## 12. Main Execution Flow

calibrate_mq2()

start all threads

while True:

blynk.run()

✔ System starts automatically
✔ Continuous monitoring
✔ Non-blocking execution

## 13. Safety & Reliability

- Thread locks prevent race conditions

- SMS cooldown prevents flooding

- Sensor averaging reduces noise

- AI adds predictive intelligence

## 14. Possible Improvements

- Add camera + fire image detection

- Cloud logging (Firebase / AWS)

- Mobile notifications instead of SMS

- Auto-start using systemd service

- Battery backup support

## 15. Conclusion

This system provides a robust, intelligent, and scalable fire detection solution, combining:

- Hardware sensors

- AI intelligence

- IoT monitoring

- GSM communication

It is suitable for homes, labs, and small industrial environments.

Youssef Ehab Fawzy

Mohamed Wanis

Mohamed Atef

Omar Khaled

Abdullah